# Visvesvaraya National Institute of Technology, Nagpur

Department of Electronics & Communication Engineering

Signals n Systems (ECL 211)

Tutorial sheet -1

Q.1 Carry out following operations on the signal shown in fig.

- x(t-2)

- x(1-t)

- [x(t) +x(1-t)] u(t-1)

- x(t) [ δ(t+3/2) - δ(t-3/2)]

- plot x(1-t/2) for the above fig.

Q.2 Find whether following signals are periodic or not

- x(t) =2cost+3cos(t/3)

- x(t) =2 cosπt + 7 cost

- x(t) =2 cos 1.5 πt + sin 3.5πt

- x[n]=cos[2n]

- x[n]=cos[2πn]

- x[n]=cos[2πt]2

Q.3  Determine whether following signals are energy or power signals and calculate their energy or power.

- x(n) =(1/2)n u(n)

- x(t) = rect (t / T0)

- x(t) =cos2 w0t

- x(t) = rect (t / T0)  cos w0t

- x(n) = u(n)

- vi)  x(t) = A e-αt u(t) ,α >0

Q.4 Find and sketch the even and odd components of the following.

- x(n) =e- (n /4 )   u(n)

- x(t) = {t   0≤t≤1

= {2-t  1≤t≤2

- x(t) =cos2 (πt /2)

Q.5 Draw the waveform for the following.

- f2(t) = -2u(t-2)

- f2(t) = -(t-2)u(t-2)

Q.6 Determine whether signals are periodic or not , determine fundamental period if periodic.

- cos (n/8) cos (nπ/8)

- sin (π+0.2n)

- 2 cos100πt +5 sin 50t